DeSoto County Mobile Source Emissions Analysis

in Support of the State Implementation Plan (SIP) Development

for the

Prepared by:

Kimley-Horn and Associates, Inc.

May 1, 2024

MISSISSIPPI DEPARTMENT OF ENVIRONMENTAL QUALITY

Table of Contents

1	Introduction	1

1.1	Planning Assumptions	3

1.2	Emissions Estimation Model	3

2	Methodology	4

2.1	MOtor Vehicle Emissions Simulator (MOVES)	6

2.2	Travel Demand Modeling	6

3	MOVES Run Specifications	7

3.1	Description	7

3.2	Scale	7

3.3	Time Spans	8

3.4	Geographic Bounds	8

3.5	Onroad Vehicles	8

3.6	Road Type	8

3.7	Pollutants and Processes	8

3.8	General Output	8

3.9	Output Emissions Detail	8

3.10	Create Input Database	9

3.11	Advanced Features	9

4	County Data Manager	9

4.1	Meteorology Data Importer	9

4.2	Source Type Population Importer	9

4.3	Age Distribution Importer	10

4.4	Vehicle Type VMT and VMT Fractions	10

4.5	Average Speed Distribution Importer	11

4.6	Road Type Distribution Importer	12

4.7	Fuel Supply, Fuel Formulation, and Fuel Usage Importer	12

4.8	Alternate Vehicle Fuel and Technology (AVFT) Importer	12

4.9	Inspection and Maintenance (I/M) Importer	12

5	Summary Results and Conclusions	12

List of Figures

Figure 1 - Memphis, TN-MS-AR 2008 8-Hour Ozone Maintenance Area	2

Figure 2 - Travel Demand Model Boundary	6

List of Tables

Table 1: MOVES Runspec Parameters	4

Table 2: MOVES County Data Manager Parameters	5

Table 3: Source Type Population for All Analysis Years	10

Table 4: Annual VMT by HPMS Type by Year	11

Table 5: Summary of Total Mobile Source Emissions	12

Table 6: Total NOx Emissions (grams and tons) by Source Type and Analysis Year	13

Table 7: Total VOC Emissions (grams and tons) by Source Type and Analysis Year	13

### Introduction

The 1990 Clean Air Act Amendments (CAAAs) and the Mississippi Transportation Conformity Rules require metropolitan transportation plans, transportation improvement programs (TIP), and transportation projects to conform to the purpose of the Mississippi State Implementation Plan (SIP). Conformity to a SIP means that federal funds will not be spent on projects that cause or contribute to any new violations of the National Ambient Air Quality Standards (NAAQS); increase the frequency or severity of NAAQS violations; or delay timely attainment of the NAAQS or any required interim milestone. The Transportation Equity Act for the 21st Century (TEA-21) and its successor legislations, the Safe, Accountable, Flexible, Efficient Transportation Equity Act:  A Legacy for Users (SAFETEA-LU), Moving Ahead for Progress in the 21st Century Act (MAP-21), the Fixing America’s Surface Transportation Act (FAST Act), and the Bipartisan Infrastructure Law (BIL) reinforce the need for coordinated transportation and air quality planning through the metropolitan planning provisions.

The purpose of this emissions analysis is to provide information about anticipated mobile source emissions to help establish emissions budgets in the SIP for the precursors of ozone for the second 10-year maintenance period. There is a SIP budget currently in place for DeSoto County, which will be superseded by the SIP budget developed as part of this analysis. The applicable SIP budget for the first 10-year maintenance period for DeSoto County can be found in Air Plan Approval and Designation of Areas; MS; Redesignation of the DeSoto County, 2008 8-Hour Ozone Nonattainment Area to Attainment, 81 Federal Register 68 (8 April 2016), p. 20543-20545.

The emissions analysis was performed according to procedures prescribed by the following federal, state and local regulations: United States Federal Register, Volume 69, p. 40004 (69 FR 40004); United States Code of Federal Regulations, Title 40, Parts 51 and 93 (40 CFR 51 and 93, i.e. Transportation Conformity Rule Requirements); the Mississippi Transportation Conformity Rules; and Metropolitan Planning Organization (MPO) Planning Regulations (23 CFR 450) implementing the Bipartisan Infrastructure Law (BIL) Requirements. Results of this emissions analysis are found in Table 5 of this report.

This report documents the process used by Kimley-Horn for the air quality analysis associated with the Desoto County SIP development. The Environmental Protection Agency’s (EPA) MOVES4 model was used to derive emissions as required by the EPA. The MOVES input files were created and modified as discussed in the pre-analysis consensus memorandum. The modeled emissions are based on a number of inputs including temperature, relative humidity, presence of inspection and maintenance programs, vehicle source type mix, vehicle age distribution, temporal distributions, average daily vehicle miles traveled (VMT), source type populations, hourly distribution, road type distribution, and average speed distribution.

The Memphis Metropolitan Planning Organization’s (MPO) Travel Demand Model was used to obtain VMT estimates for the portion of DeSoto County in the maintenance area. For more information on the methodology and assumptions utilized in the development of the Travel Demand Model, please refer to the Model Documentation available upon request from the Memphis MPO.

Ozone (O3)

EPA designated Memphis, TN-MS-AR as a 2008 8-hour ozone marginal non-attainment area effective July 20, 2012. The final ruling was published in the Federal Register (77 FR 30088) on May 21, 2012. Included in this designation were Crittenden County, AR in the West Memphis MPO area and Shelby County, TN, and the portion of DeSoto County, MS in the Memphis Urban Area MPO boundary at the time of the designation.

On May 9, 2016, EPA redesignated the DeSoto County portion of the area from non-attainment to attainment for the 2008 8-hour ozone standard. The final ruling was published in the Federal Register (81 FR 20543) on April 8, 2016. Similar redesignations from non-attainment to attainment for the Tennessee and Arkansas portions of the area were also made in 2016. Figure 1 shows the TN-MS-AR maintenance area for the 2008 8-hour ozone standard.

Figure 1 - Memphis, TN-MS-AR 2008 8-Hour Ozone Maintenance Area

EPA designated DeSoto County, MS as Attainment/Unclassifiable for the 2015 8-hour Ozone NAAQS effective January 16, 2018. The final ruling was published in the Federal Register (82 FR 54232) on November 16, 2017. Also included in this designation were Crittenden County, AR in the West Memphis MPO area and Shelby County, TN in the Memphis Urban Area Metropolitan Planning Organization (MPO). Therefore, demonstration of conformity is not required for the 2015 8-hour Ozone NAAQS.

In July 2012, EPA issued the companion guidance to the Conformity Rule that addresses ozone and air quality standards. The guidance further clarified how conformity determinations and the regional emissions analyses that support them are completed in existing and new non-attainment and maintenance areas. The guidance noted that states in a multi-state area have the option of submitting SIPs with budgets for their own portion of the area that, when taken together, meet the applicable Clean Air Act requirement. Where states have done so, and EPA has found such budgets adequate or approved, the MPO or MPOs in each state with such budgets can determine conformity completely independently of the other states.

Further, all affected agencies need to be included in the decision-making process for the area as required by the conformity rule described in the Code of Federal Regulations, Title 40, Part 93, Section 105 (40 CFR 93.105). Shelby County, Tennessee; DeSoto County, Mississippi; and Crittenden County, Arkansas have budgets of their own. During the original interagency consultation process for the development of the existing SIP, it was decided that DeSoto County would not be judged within the overall SIP budget established for Shelby County but would instead be subject to an independent conformity demonstration. Therefore, this analysis is only for the DeSoto County portion of the 2008 8-hour ozone maintenance area.

#### Planning Assumptions

The Memphis MPO Travel Demand Model was developed with consultation and input from state and local transportation agencies. The Moving Together 2050 RTP provides the appropriate level of detail required by 40 CFR 93.106 of the conformity regulations. The highway projects in the Moving Together 2050 RTP and the current FY 2023-26 TIP are financially constrained for each horizon year in terms of capital, operations and maintenance costs. The emissions analysis is based on assumptions derived from estimates of current and future population, employment, travel, and congestion. As part of the Moving Together 2050 RTP and the FY 2023-26 TIP conformity determination, past assumptions have been discussed with various local, state and federal agencies for their continued validity and updated whenever necessary. Detailed planning assumptions are presented in Section 2 of this report.

#### Emissions Estimation Model

Mobile source emissions estimates for the ozone season (summer) were developed using EPA's Motor Vehicle Emission Simulator, MOVES4 (January 2024), and travel estimates from the current Memphis MPO TDM. The Memphis MPO, Tennessee Department of Transportation (TDOT), Mississippi Department of Transportation (MDOT), and Shelby County Health Department provided the most current data available for emissions calculations. The Federal Highway Administration (FHWA) and EPA's Region 4 Air Planning Branch provided assistance and guidance as well.

### Methodology

The emissions inventory development and emissions projection discussion below identifies procedures used to obtain emissions. Table 1 summarizes the settings used in the MOVES run specification file. Table 2 lists the assumptions used in the MOVES County Data Manager. Further details on the use of MOVES are found in the following sections.

Table 1: MOVES Runspec Parameters

| MOVES Runspec Parameter | Settings |
| --- | --- |
| MOVES4.0.1, January 2024 | Database version - MOVESDB20240104 |
| Scale | County, Inventory |
| Time Span | Time aggregation = Hour 1 month representing summer conditions (July) for ozone All hours of the day selected Weekdays only |
| Geographic Bounds | 1 custom domain - DeSoto County (partial) |
| Vehicles/Equipment | All valid source types and fuel types, including gasoline, diesel, E-85, and CNG |
| Road Type | All road types including off-network |
| Pollutants and Processes | NOx, VOCs, total gaseous hydrocarbons, non-methane hydrocarbons |
| General Output | Units = grams, joules, miles |
| Output Emissions | Time = hour, geographic = county, on-road emissions rates by road type and source use type |
| Advance Performance | None |

Table 2: MOVES County Data Manager Parameters

| County Data Manager Input | Data Source |
| --- | --- |
| Meteorology Data | Local data was obtained from National Oceanic and Atmospheric Administration (NOAA) at the Memphis International Airport. This data represents a three-year average of hourly temperatures and humidity. |
| Source Type Population | Local and state information is currently under development and as such is unavailable. As a result, MOVES4 default data was used for all source types. The MOVES Technical Guidance provides guidance on how to adjust default data with local VMT data. |
| Age Distribution | 2020 Data prepared by the TDOT Forecasting Office and MOVES3 default data for source types 51, 52, 53, 61, and 62 for Shelby County was applied for vehicle source types in DeSoto County. |
| Vehicle Type VMT – HPMS Vehicle Type Year VMT | Local data obtained from Memphis Travel Demand Model with EPA’s daily to annual VMT converter applied |
| Vehicle Type VMT – Monthly VMT Fraction | Data obtained from EPA’s daily to annual VMT converter |
| Vehicle Type VMT – Daily VMT Fraction | Data obtained from EPA’s daily to annual VMT converter |
| Vehicle Type VMT – Hourly VMT Fraction | Local data obtained from Memphis Travel Demand Model |
| Average Speed Distribution | Local data obtained from Memphis Travel Demand Model |
| Road Type Distribution | Local data obtained from Memphis Travel Demand Model |
| Fuel Supply/Fuel Formulation/Fuel Usage | Default data from MOVES4 |
| Alternate Vehicle Fuel and Technology | Default modified to exclude compressed natural gas (CNG) transit buses. In addition, Shelby County electric vehicle (EV) distribution for passenger vehicles was used in the absence of DeSoto County data. The AVFT tool in MOVES4 was used to forecast existing distributions to 2036. |
| I/M Programs | No I/M used |

#### MOtor Vehicle Emissions Simulator (MOVES)

The Clean Air Act (CAA) requires EPA to regularly update its mobile source emission models. EPA continuously collects data and measures vehicle emissions to make sure the Agency has the best possible understanding of mobile source emissions. This assessment, in turn, informs the development of EPA’s mobile source emission models. MOVES represents the Agency’s most up-to-date assessment of mobile source emissions. MOVES also incorporates several changes to EPA’s approach to mobile source emission modeling based upon recommendations made to the Agency by the National Academy of Sciences. MOVES4, which represents the Agency’s most up-to-date assessment of on-road mobile source emissions, was used for this analysis.

#### Travel Demand Modeling

The Memphis Urban Area Travel Demand Model was developed in consultation with states, local transportation agencies and local jurisdictions. Its boundary includes all of Fayette, Shelby, and Tipton County in Tennessee; DeSoto, Marshall, Tate, and Tunica County in Mississippi; and Crittenden County, Arkansas (see Figure 2).

Figure 2 - Travel Demand Model Boundary

The Travel Demand Model’s base year is 2016. The Travel Demand Model is a standard four step model with trip generation, trip distribution, mode choice, and trip assignment components. The Travel Demand Model includes freight and transit subcomponents. Year 2016 count data from TDOT, MDOT, and ARDOT, as well as counts collected by the MPO were used in the base year model validation and calibration effort. Socioeconomic data was forecasted to the year 2050 as a part of the most recent RTP. There are 2,823 internal and 54 external traffic analysis zones in the Travel Demand Model. The Travel Demand Model roadway network includes all interstates, freeways and expressways, principal arterials, minor arterials, and collectors, as well as some local streets. Although model approval is a joint process between the MPO and the appropriate state review agencies, TDOT was the primary agency responsible for approval of the travel demand model. The TDM was reviewed by the TDOT Long Range Planning Division using the Minimum Travel Demand Model Calibration and Validation Guidelines for the State of Tennessee, and concurrence on use of the model for the Memphis MPO’s regional planning efforts was received on May 15, 2020. Additional Travel Demand Model development documentation is available from the Memphis MPO upon request.

Model Data Adjustments

The Travel Demand Model results were compared with the 2016 Highway Performance Monitoring System (HPMS) data obtained from the Mississippi Department of Transportation (MDOT) for DeSoto County. The annual VMT shown in the HPMS data was compared to the total 2016 DeSoto County VMT shown in the Travel Demand Model. This comparison yielded a HPMS/TDM ratio that was used to scale the VMT output from the model to match documented conditions more closely. This scaled VMT was used in the development of the HPMS vehicle type VMT.

Pre-Processing

Information was gathered from the Travel Demand Model to generate the average speed distribution, road type distribution, hourly VMT distribution, HPMS vehicle type VMT, and ramp fraction. To streamline this process, a pre-processor was used to perform many of the calculations and disaggregations needed to produce MOVES-ready spreadsheets for each input. MOVES spreadsheet templates for each input type were developed for the identified model years. Following this, a script was used to pull data from the model and perform the required calculations. MOVES files generated through this exercise were then applied directly in the County Data Manager. Additional information on the data sources used and needed data manipulation is contained in Sections 3 and 4. The analysis was performed using the inventory method. As a result, no post-processing was needed for the MOVES data output.

### MOVES Run Specifications

The MOVES4.0.1 software was released by EPA in January 2024. This software uses a graphical user interface with a set of input categories. A Runspec was developed that stores the input values for these categories. The values and information included in the Runspecs developed for this analysis are explained in more detail in the following sections.

#### Description

The information in this category is used to distinguish the individual Runspecs. For this analysis, the description is used to introduce the purpose for the analysis, the area being studied, and the year of analysis (i.e., 2017 and 2036).

#### Scale

This input window is used to detail the information needed for the domain/scale of the analysis as well as the calculation type. The county level was selected as the domain for this effort since it is the appropriate level for use in SIP and regional conformity analysis. The inventory method was chosen for the calculation type.

#### Time Spans

The Time Spans input window has a variety of different timescale inputs that are used for understanding the level of temporal aggregation being used in the analysis. The time aggregation level was specified as hours, based on guidance from EPA/FHWA for the preferred aggregation level for conformity runs. The years 2017 and 2036 were chosen for the analysis years. Each year was done within a different Runspec. Pollutant analysis for DeSoto County is for 8-hour ozone conditions. July was chosen as the month to best represent summer ozone conditions. Weekdays were selected as the representative day type since they are considered to be the worst-case type when compared with weekends. All hours of the day were included in the analysis to represent conditions over a full 24-hour period.

#### Geographic Bounds

This analysis consisted of the portion of DeSoto County in the maintenance area.

#### Onroad Vehicles

This input window allows the user to specify which fuel and vehicle types are present within the transportation network. There are thirteen different vehicle classes (referred to as source use types) and five different fuel types. All possible vehicle and fuel types were considered.

#### Road Type

The MOVES software incorporates five different roadway types: off-network, rural restricted access, rural unrestricted access, urban restricted access, and urban unrestricted access. Expressways and freeways in the region are considered as restricted access facilities. For this analysis, all five roadway types were considered. Off-network emissions are intended to account for vehicle starts and evaporative emissions for parked vehicles. While these emissions are not captured through the information in the regional Travel Demand Models, default values can be used to assess their impacts.

#### Pollutants and Processes

This input window allows the user to specify different pollutants and processes that are desired for modeling. Since the purpose of this analysis is to assess emissions relating to the 2008 8-hour ozone standard, the Runspec includes Oxides of Nitrogen (NOx), Volatile Organic Compounds (VOC), Non-Methane Hydrocarbons (NMHC), and Total Gaseous Hydrocarbons (THC). Output databases were adjusted to include results by pollutant and process types.

#### General Output

Output for the MOVES program is stored in a user-created database. Output databases were created for each of the different analysis years and Runspec conditions. The unit of measure used in the output database were grams, joules, and miles.

#### Output Emissions Detail

The time measurement for this analysis was set as hourly, and the location was set for the county level. To assist with post-processing aggregation, the source use type information was included with the output.

#### Create Input Database

Inputs for the MOVES program are stored in user-created databases. Input databases were created for each of the different analysis years and Runspec conditions. Files are added to the input databases using the County Data Manager, which is discussed in further detail in Section 4.

#### Advanced Features

The MOVES software includes input windows where provisions can be specified for specific strategies such as on-road retrofit and rate of progress emissions. Since these strategies are not being applied in this location, no information was entered for this section.

### County Data Manager

MOVES4 allows for input databases to be created and data entered through the Runspec level of the model. Once the base parameters have been established for a given MOVES Runspec, the Create Input Database tab can be used to create a databased and enter locally specific data. Input provided in Excel spreadsheet format can be referenced using this tool, which converts the data to HeidiSQL format and incorporates it into the MOVES analysis. For DeSoto County, locally specific data could consist of data used for the entire region, statewide, or county-level (DeSoto County, MS or Shelby County, TN) data. The following sections detail these input criteria, and the methodology and assumptions used to arrive at the information entered for each.

#### Meteorology Data Importer

Meteorological data for this area is consistent with the local data from National Oceanic and Atmospheric Administration (NOAA) data at the Memphis International Airport. A three-year average of data for hourly temperatures and humidity was obtained from NOAA from the years 2021-2023. This analysis assumes the meteorological data at this location can be applied to the entire modeled area.

#### Source Type Population Importer

This importer provides the user with the opportunity to enter vehicle population data sorted by the 13 MOVES vehicle source types. Local and state information is currently under development and, as such, is unavailable for DeSoto County. As a result, default data from MOVES was used as a starting point for all source types. The MOVES Technical Guidance offers guidance on how to leverage default population and VMT data alongside local VMT data to arrive at an estimate of the local population. This guidance suggests comparing the ratio of default source type population to default VMT to the local VMT to arrive at an estimate of local source type population. To do this, default source type population and default VMT data was obtained from MOVES, while local VMT data was obtained from the Travel Demand Model. The adjusted values more accurately reflect the local source type population values for DeSoto County. Table 3 shows the source type population used for this analysis.

Table 3: Source Type Population for All Analysis Years

| Source Type | Source Type ID | Analysis Year |  |
| --- | --- | --- | --- |
|  |  | 2017 | 2036 |
| Motorcycle | 11 | 1,424 | 2,506 |
| Passenger Car | 21 | 108,427 | 184,107 |
| Passenger Truck | 31 | 32,633 | 55,301 |
| Light Commercial Truck | 32 | 3,042 | 5,155 |
| Intercity Bus | 41 | 30 | 67 |
| Transit Bus | 42 | 10 | 20 |
| School Bus | 43 | 44 | 95 |
| Refuse Truck | 51 | 35 | 58 |
| Single Unit Short-haul Truck | 52 | 4,137 | 8,635 |
| Single Unit Long-haul Truck | 53 | 182 | 381 |
| Motor Home | 54 | 599 | 1,176 |
| Combination Short-haul Truck | 61 | 874 | 1,638 |
| Combination Long-haul Truck | 62 | 587 | 1,084 |
| Total for Each Analysis Year |  | 152,024 | 260,224 |

#### Age Distribution Importer

The Age Distribution Importer allows the user to provide vehicle age distribution data sorted by the MOVES vehicle source types. Vehicle age distribution is divided into 30 years based on vehicle model years. For each vehicle type, the sum of all age distributions will equal one. Local data for DeSoto County was not available. Shelby County specific data for 2020 was available from the TDOT Long Range Planning Forecasting Office report “Methodology for Developing Input Datasets for the 2020 MOVES Model,” January 2022. This information was used as a surrogate for DeSoto County for all analysis years. Shelby County default data from MOVES was used for source types 51, 52, 53, 61, and 62 to be consistent with the methodology used in Shelby County for the development of source type population to better represent local conditions. Additional information on the Shelby County source type population development process can be found in Technical Report #8A – Air Quality Conformity Report, Shelby County of the Moving Together 2050 RTP on the MPO’s website.

#### Vehicle Type VMT and VMT Fractions

This data importer asks the user for the VMT in the study area by HPMS vehicle class type, hourly VMT distributions, daily VMT distributions, and monthly VMT distributions. The HPMS vehicle class VMT is asked for on a daily or annual basis, as selected by the user. To determine this information, data can be pulled from available Travel Demand Models or from regional HPMS data.

HPMS Vehicle Class VMT

The HPMS vehicle class VMT was estimated using the pre-processor developed within the Travel Demand Model. The Travel Demand Model classifies vehicles into automobiles, single unit trucks, and combination unit trucks. The three vehicle classes in the model were divided into the five HPMS vehicle class types through the pre-processor. The HPMS vehicle types are motorcycles, light duty vehicles, buses, single unit trucks, and combination unit trucks. Since the Travel Demand Model produces daily weekday volumes, EPA’s Daily to Annual HPMS VMT conversion tool was used to convert these daily VMT numbers to annual values. The annual vehicle miles traveled by HPMS vehicle source type for the study area are shown in Table 4.

Table 4: Annual VMT by HPMS Type by Year

| Source Type | HPMS Type ID | Analysis Year |  |
| --- | --- | --- | --- |
|  |  | 2017 | 2036 |
| Motorcycle | 10 | 5,078,619 | 8,455,851 |
| Passenger Car and Light Truck | 25 | 1,741,111,157 | 2,898,932,890 |
| Intercity, Transit, and School Bus | 40 | 1,681,766 | 2,967,741 |
| Single Unit Truck | 50 | 65,320,847 | 115,268,955 |
| Combination Unit Truck | 60 | 96,504,936 | 163,414,973 |
| Total for Each Analysis Year |  | 1,909,697,324 | 3,189,040,411 |

Daily VMT Fraction

Local data was unavailable; therefore, the analysis used dayVMTFraction data obtained from EPA’s Daily to Annual VMT converter.

Monthly VMT Fraction

Local data was unavailable; therefore, the analysis used monthVMTFraction data obtained from EPA’s Daily to Annual VMT converter.

Hourly VMT Fraction

The hourly VMT fraction was determined using the pre-processor developed within the Travel Demand Model. To produce the information needed for the MOVES input file, the three vehicle classes in the model were expanded to the 13 MOVES vehicle source types. In addition, the four time of day periods in the model were expanded to represent each hour of the day. The default mix of off-network hourly distribution percentages was used for all vehicle classes.

#### Average Speed Distribution Importer

This importer gives the user the opportunity to enter locally specific average speed data, disaggregated by vehicle source type, road type, weekday/weekend, and hour of the day. The MOVES model uses 16 speed bins, dividing speed distributions into a fraction of driving within each speed bin for each of the criteria listed previously. The average speed distribution was determined using the pre-processor developed within the Travel Demand Model. The vehicle classes in the model were expanded to the 13 MOVES vehicle source types, and the hourly distribution was expanded from the four time periods in the model to each hour of the day.

#### Road Type Distribution Importer

The road type distribution importer can be used to incorporate locally specific roadway distribution information. The road type distribution was determined using the pre-processor developed within the Travel Demand Model. The vehicle classes in the model were expanded to the 13 MOVES vehicle source types.

#### Fuel Supply, Fuel Formulation, and Fuel Usage Importer

The fuel supply, fuel formulation, and fuel usage importers are used to input locally-specific fuel properties into the model. Default data from MOVES for DeSoto County was used for this category.

#### Alternate Vehicle Fuel and Technology (AVFT) Importer

This importer considers the AVFT vehicle mix in the overall population. If no information is entered for AVFT, MOVES assumes a default mix of fuels for each source type. The MOVES default mix for DeSoto County assumes a small percentage of compressed natural gas (CNG) transit buses. Since there are no alternative fuel vehicles in the transit fleet for DeSoto County, the default value for this file was modified to reflect only diesel and gasoline fuel types for existing transit buses (source type 42). Additionally, the default EV population in MOVES is based on national averages, so local information should be used if available. However, existing electric vehicle (EV) registration data is not readily available for DeSoto County. Existing registration data for Shelby County for 2020 was used as a surrogate for DeSoto County to develop a breakdown of electric vehicles for source type 21 (passenger vehicles). This distribution will be used for the base year of 2017. The AVFT Tool in MOVES4 was used to forecast the existing distribution to 2036. Within the AVFT Tool, the Proportional Method was used for source types 21 and 42, while the National Average Method was used for all other source types to project the distributions to 2036.

#### Inspection and Maintenance (I/M) Importer

This importer allows local inspection and maintenance data to be entered for the study area. There is no existing or proposed I/M program in DeSoto County, so no further data was entered.

### Summary Results and Conclusions

Results from this analysis are summarized in Table 5. Additionally, Tables 6 and 7 show total NOx and VOC emissions for each analysis year by the 13 MOVES vehicle source types.

Table 5: Summary of Total Mobile Source Emissions

| Pollutant | Emissions (tons/day) |  |
| --- | --- | --- |
|  | 2017 | 2036 |
| Volatile Organic Compounds (VOC) | 3.155 | 1.628 |
| Oxides of Nitrogen (NOx) | 5.930 | 1.252 |

Table 6: Total NOx Emissions (grams and tons) by Source Type and Analysis Year

| Source Type | Source Type ID | Analysis Year |  |
| --- | --- | --- | --- |
|  |  | 2017 | 2036 |
| Motorcycle | 11 | 12,815 | 19,062 |
| Passenger Car | 21 | 1,527,727 | 163,501 |
| Passenger Truck | 31 | 1,009,218 | 88,620 |
| Light Commercial Truck | 32 | 107,602 | 11,207 |
| Intercity Bus | 41 | 18,700 | 9,001 |
| Transit Bus | 42 | 4,055 | 1,394 |
| School Bus | 43 | 10,151 | 2,998 |
| Refuse Truck | 51 | 17,718 | 6,609 |
| Single Unit Short-haul Truck | 52 | 489,792 | 144,526 |
| Single Unit Long-haul Truck | 53 | 30,215 | 7,505 |
| Motor Home | 54 | 42,603 | 12,074 |
| Combination Short-haul Truck | 61 | 674,067 | 209,095 |
| Combination Long-haul Truck | 62 | 1,435,047 | 460,117 |
| Total (grams/day) |  | 5,379,710 | 1,135,709 |
| Total (tons/day) |  | 5.930 | 1.252 |

Table 7: Total VOC Emissions (grams and tons) by Source Type and Analysis Year

| Source Type | Source Type ID | Analysis Year |  |
| --- | --- | --- | --- |
|  |  | 2017 | 2036 |
| Motorcycle | 11 | 48,906 | 67,841 |
| Passenger Car | 21 | 1,753,260 | 963,428 |
| Passenger Truck | 31 | 703,622 | 296,925 |
| Light Commercial Truck | 32 | 61,105 | 24,602 |
| Intercity Bus | 41 | 2,034 | 2,660 |
| Transit Bus | 42 | 627 | 538 |
| School Bus | 43 | 1,650 | 574 |
| Refuse Truck | 51 | 1,565 | 611 |
| Single Unit Short-haul Truck | 52 | 141,687 | 62,317 |
| Single Unit Long-haul Truck | 53 | 7,097 | 2,808 |
| Motor Home | 54 | 23,451 | 12,713 |
| Combination Short-haul Truck | 61 | 37,734 | 15,063 |
| Combination Long-haul Truck | 62 | 79,487 | 26,846 |
| Total (grams/day) |  | 2,862,225 | 1,476,926 |
| Total (tons/day) |  | 3.155 | 1.628 |

Based on the results from the MOVES analysis, the trend is a general reduction in emissions of VOC and NOx. The reduction can be attributed to a variety of factors including the expected future shift to EVs for all vehicle types, the implementation of stricter standards for gasoline and diesel, and improving vehicle technology over time resulting in fewer emissions.

The current Mississippi State Implementation Plan (SIP) established emission budgets for transportation conformity purposes based on the year 2027 for the first 10-year maintenance period. The budgets for NOx and VOC for 2027 are 8.000 tons/day and 5.000 tons/day, respectively. The results of this analysis show that the expected emissions for 2036 will continue to conform to the budgets set in the current SIP. Based on this analysis, new budgets will be established for the second 10-year maintenance period that will supersede the existing budgets for all years beyond 2027.

https://www.gpo.gov/fdsys/pkg/FR-2016-04-08/pdf/2016-08155.pdf

Using MOVES to Prepare Emission Inventories in State Implementation Plans and Transportation Conformity:  MOVES4 Technical Guidance, EPA, August 2023. https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=P101862T.pdf

Highway Performance Monitoring System (HPMS)

https://memphismpo.org/plans/regional-transportation-plan-rtp/moving-together-2050-regional-transportation-plan

https://www.epa.gov/moves/tools-develop-or-convert-moves-inputs